EHS事故报告和调查处理制度

EHS Accident Reporting and Investigation System

内部使用

在使用前，请和文档编写人员确认本文档为最新版本。

未经上海暻熠数据科技有限公司、上海暻耀网络技术有限公司书面许可，其它任何机构或个人不可传阅、引用或复制本文件的内容。

Internal use

Before using, check with the documentation writer to make sure this document is the latest version.

Without the written permission of Shanghai Jingyi Data Technology Co., Ltd., Shanghai Jingyao internet Technology Co., Ltd., no other organization or individual is allowed to circulate, quote or copy the content of this document.

| 文件编号Documents | IP-PJ-EHS-21 |
| --- | --- |
| 版 本Edition | V1.0 |
| 文件维护部门File maintenance department | 上海浦江数据中心 |
| 会签部门Document Signing department | 上海浦江数据中心 |
| 执行范围Scope of execution | 上海浦江数据中心 |

文件修订记录

Document revision record

| 版本号 Edition | 版本日期 Date | 修订内容 Revised Content | 修订人员 Revision personnel | 核准员 Approver | 备注 Remarks |
| --- | --- | --- | --- | --- | --- |
| V1.0 | 2025-04-16 | 首次修订 | 楚靖岳 | 陆磊 |  |

目录 Content

1 目的Purpose	4

2 适用范围Scope of application	4

3 规范性引用文件Reference documents	4

4 术语和定义Terms and definitions	4

5 职责Responsibility	5

5.1 各部门/岗位/业务线 All departments/ positions/ business lines	5

5.2 主要负责人 Main responsible person	6

5.3 EHS管理员 EHS officer	6

6 程序 Procedures	6

6.1 EHS事故事件管理通用要求 General requirements for EHS incident management	6

6.2 EHS事故事件分级分类 Classification and grading of EHS incident	7

6.3 EHS事故事件报告 EHS incident report	11

6.4 EHS事故事件的应急响应行动 Emergency response actions for EHS incidents	13

6.5 EHS事故事件调查和报告 EHS incidents investigation and report	13

6.6 EHS事故事件学习和审查 EHS incidents learning and review	14

6.7 OSHA可记录事件管理 OSHA recordable incident management	15

6.8 沟通与培训Communication and training	21

7 附件Attachments	22

1 目的Purpose

本制度旨在建立上海暻熠数据科技有限公司、上海暻耀网络技术有限公司（以下简称“上海浦江数据中心”）安全生产责任制度。目标包括确保遵守国家和地方法律法规、国外相关法规、客户要求以及上海浦江数据中心内部要求，明确并落实各岗位人员的安全生产责任，预防和减少生产安全事故的发生，降低运营EHS风险，并促进持续改进。

The purpose of the System is to establish the management process of EHS accident reporting and investigation of Shanghai Jingyi Data Technology Co., Ltd., Shanghai Jingyao internet Technology Co., Ltd.. (hereinafter referred to as "SHANGHAI Pujiang Data Center"). The objectives include ensuring compliance with national and local laws and regulations, related foreign regulations, customer requirements and internal requirements. To reduce operational EHS risks and to promote continuous improvement.

2 适用范围Scope of application

适用于上海浦江数据中心，以及所有与EHS有关事项的管理职能部门。

This System applies to the SHANGHAI PUJIANG Data Center and all management functions involved in EHS (Environment, Health, and Safety) matters.

3 规范性引用文件Reference documents

《生产安全事故报告和调查处理条例》（中华人民共和国国务院令第493号）

Regulations on the Reporting, Investigation and Handling of Production Safety Accidents (Decree No. 493 of the State Council of the People's Republic of China)

《生产安全事故应急条例》（中华人民共和国国务院令第708号）

Regulations on Emergency Response to Production Safety Accidents (Decree No. 708 of the State Council of the People's Republic of China)

《生产安全事故信息报告和处置办法》 （国家安全生产监督管理总局令第21号）

Measures for the Reporting and Handling of Information on Production Safety Accidents (Decree No. 21 of the State Administration of Work Safety)

《Recording and Reporting Occupational Injuries and Illnesses记录和报告职业伤

害和疾病》（29 CFR 1904, 标准号：29号美国联邦法规第1904部分）

Recording and Reporting Occupational Injuries and Illnesses (29 CFR 1904, Standard Number: Part 1904 of Title 29 of the Code of Federal Regulations)

4 术语和定义Terms and definitions

4.1特别重大事故Extraordinarily serious accident

是指造成30人以上死亡，或者100人以上重伤（包括急性工业中毒，下同），或者1亿元以上直接经济损失的事故。

It refers to accidents that result in the death of more than 30 people, serious injuries (including acute industrial poisoning, the same below) to more than 100 people, or direct economic losses exceeding 100 million yuan.

4.2重大事故 Major accident

是指造成10人以上30人以下死亡，或者50人以上100人以下重伤，或者5000万元以上1亿元以下直接经济损失的事故。

It refers to accidents that cause the death of more than 10 but fewer than 30 people, serious injuries to more than 50 but fewer than 100 people, or direct economic losses of more than 50 million yuan but less than 100 million yuan.

4.3较大事故 Major accident at a medium-level

是指造成3人以上10人以下死亡，或者10人以上50人以下重伤，或者1000万元以上5000万元以下直接经济损失的事故。

It refers to accidents that result in the death of more than 3 but fewer than 10 people, serious injuries to more than 10 but fewer than 50 people, or direct economic losses of more than 10 million yuan but less than 50 million yuan.

4.4一般事故 Ordinary-level accident

是指造成3人以下死亡，或者10人以下重伤，或者1000万元以下直接经济损失的事故。

It refers to accidents that result in the death of fewer than 3 people, serious injuries to fewer than 10 people, or direct economic losses of less than 10 million yuan.

5 职责Responsibility

## 5.1 各部门/岗位/业务线 All departments/ positions/ business lines

5.1.1负责发生EHS相关事故事件后的汇报和现场应急处置。

Be responsible for reporting and on-site emergency response after the occurrence of EHS-related incidents.

5.1.2组织内部EHS事故事件管理培训以及参加EHS管理负责人组织的相关培训。

Organize internal EHS incident management training and participate in relevant training organized by the EHS management personnel.

5.1.3参与EHS事故事件调查，编制事故调查报告以及后续问题/隐患整改。

Participate in the investigation of EHS incidents, prepare accident investigation reports, and implement corrective actions for identified problems and potential hazards.

## 5.2 主要负责人 Main responsible person

5.2.1及时、如实报告生产安全事故。

Report production safety accidents in a timely and truthful manner.

## 5.3 EHS管理员 EHS officer

5.3.1负责制定EHS事故事件管理制度。

Be responsible for formulating the EHS incident management System.

5.3.2负责EHS事故事件管理全过程监督管理。

Be responsible for the supervision and management of the entire process of EHS incident management.

5.3.3为不同部门/岗位/业务线人员组织EHS事故事件管理培训。

Organize EHS incident management training for personnel from different departments, positions, and business lines.

5.3.4组织各部门/岗位/业务线人员开展EHS事故事件调查，并对EHS事故事件进行记录。

Organize personnel from all departments, positions, and business lines to conduct EHS incident investigations and document the incidents.

5.3.5 组织相关供应商参加事故事件调查。

Organize relevant suppliers to participate in incident investigations.

5.3.6与当地政府相关部门沟通，对相关EHS事故事件进行汇报以及调查结果的汇报。

Communicate with relevant local government departments, report relevant EHS incidents, and provide investigation results.

6 程序 Procedures

## 6.1 EHS事故事件管理通用要求 General requirements for EHS incident management

6.1.1事故报告应当及时、准确、完整，数据中心对事故不得迟报、漏报、谎报或者瞒报。事故调查处理应当坚持实事求是、尊重科学的原则，及时、准确地查清事故经过、事故原因和事故损失，查明事故性质，认定事故责任，总结事故教训，提出整改措施，并对事故责任者依法追究责任。

Accident reports shall be timely, accurate, and complete. The data center shall not delay, omit, falsify, or conceal accident reports. Accident investigation and handling shall adhere to the principles of seeking truth from facts and respecting science. It is necessary to promptly and accurately identify the process, causes, and losses of the accident, determine the nature of the accident, assign responsibility for the accident, summarize lessons learned, propose corrective actions, and hold those responsible for the accident accountable in accordance with the law.

6.1.2 事故调查组成员应当具有事故调查所需要的知识和专长，并与所调查的事故没有直接利害关系。

Members of the accident investigation team shall possess the knowledge and expertise required for the investigation and shall have no direct interest in the accident under investigation.

6.1.3事故发生单位负责人（数据中心主要负责人）接到事故报告后，应当立即启动事故相应应急预案，或者采取有效措施，组织抢救，防止事故扩大，减少人员伤亡和财产损失。

Upon receiving the accident report, the main responsible person shall immediately activate the corresponding emergency response plan or take effective measures to organize rescue efforts, prevent the accident from escalating, and reduce casualties and property losses.

6.1.4 事故事件调查结束后，数据中心需要对事故事件原因进行分析，列出整改项，并明确整改责任人、整改完成时间以及整改完成情况等。并将事故事件汲取的经验教训，对相关人员开展培训。

After the incident investigation is completed, the data center shall analyze the cause of the incident, list corrective actions, and clearly define the responsible person, completion deadline, and status of corrective actions. Additionally, the data center shall conduct training for relevant personnel and share lessons learned from the incident.

## 6.2 EHS事故事件分级分类 Classification and grading of EHS incidents

6.2.1根据生产安全事故造成的人员伤亡或者直接经济损失，事故一般分为以下等级：

Based on the casualties or direct economic losses caused by production safety accidents, accidents are generally classified into the following grades:

- 特别重大事故，是指造成30人以上死亡，或者100人以上重伤（包括急性工业中毒，下同），或者1亿元以上直接经济损失的事故；

Extraordinarily Serious Accidents: Accidents that result in the death of more than 30 people, serious injuries (including acute industrial poisoning, the same below) to more than 100 people, or direct economic losses exceeding 100 million yuan.

- 重大事故，是指造成10人以上30人以下死亡，或者50人以上100人以下重伤，或者500万元以上1亿元以下直接经济损失的事故；

Major Accidents: Accidents that cause the death of more than 10 but fewer than 30 people, serious injuries to more than 50 but fewer than 100 people, or direct economic losses of more than 50 million yuan but less than 100 million yuan.

- 较大事故，是指造成3人以上10人以下死亡，或者10人以上50人以下重伤，或者1000万元以上5000万元以下直接经济损失的事故；

Major accident at a medium-level: Accidents that result in the death of more than 3 but fewer than 10 people, serious injuries to more than 10 but fewer than 50 people, or direct economic losses of more than 10 million yuan but less than 50 million yuan.

- 一般事故，是指造成3人以下死亡，或者10人以下重伤，或者1000万元以下直接经济损失的事故。

Ordinary-level accident: Accidents that result in the death of fewer than 3 people, serious injuries to fewer than 10 people, or direct economic losses of less than 10 million yuan.

6.2.2上述为《生产安全事故报告和调查处理条例》对于生产安全事故的分级。结合行业及生产安全事故事件最佳管理实践，上海浦江数据中心在发生上述不同级别的事故之后，需要上报至万国数据集团以及当地政府相关管理职能部门，并留存好相关记录，以备政府相关管理职能部门或其他相关方检查。

The above is the classification of production safety accidents as outlined in the "Regulations on the Reporting, Investigation, and Handling of Production Safety Accidents." In line with industry standards and best practices for production safety incident management, the SHANGHAI PUJIANG Data Center must report accidents of the aforementioned levels to GDS Group and the relevant management departments of the local government. Additionally, relevant records must be properly maintained for inspection by the relevant government management departments or other stakeholders.

6.2.3 除生产安全事故事件以外，结合跨国企业EHS事故事件分级管理经验，可参考如下表1：

EHS事故事件分级分类。

In addition to production safety incidents, drawing on the hierarchical management experience of EHS incidents in multinational enterprises, reference can be made to Table 1 below: Classification and Grading of EHS Incidents.

表1 EHS事故事件分级分类

Table 1 Classification and Grading of EHS Incident

| 事故事件分级 Incident Grading | 重大事件 Significant Incident | 严重事件 Serious Incident | 较大事件 Major Incident | 轻微事件 Minor Incident |
| --- | --- | --- | --- | --- |
| 安全管理相关 Related to safety management | 工作相关的死亡 Work-related death | 工作相关的永久性伤害 Work-related permanent injuries | OSHA或本地可记录伤害/疾病 OSHA or local recordable injuries/illnesses | 需要急救的伤害 Injuries requiring first aid |
|  |  | 工作相关的医疗处理响应事件，需要住院和医疗治疗 Work-related medical treatment response incidents that require hospitalization and medication | 工作相关的医疗处理响应事件，需要外部应急响应者协助，如需要到医院进行处理 Work-related medical treatment response incidents requiring assistance from external emergency responders, such as hospital treatment | 工作相关的医疗，在现场应急响应者的能力范围之内，现场简单处理即可恢复 Work-related medical treatment within the capabilities of on-site emergency responders, recoverable with simple on-site treatment |
| 环境 Environment | 影响到社区环境，在数据中心的运营业务区域之外，中断公共服务，或触发社区保护措施。（如废气排放或者厂界噪声严重影响数据中心周边相关方） Impacts the community environment, occurs outside the operational area of the data center, disrupts public services, or triggers community protection measures (e.g., exhaust emissions or noise at the site boundary significantly affecting surrounding stakeholders) | 缺失排污许可证（后续如需办理）或多个环境许可事项 Missing pollutant discharge permit (to be applied for later if needed) or multiple environmental permit issues | 许可超标但未提交法规要求的报告 Exceeding permit limits but failing to submit reports as required by regulations | 超过内部管理排放限值 Exceeding internal management emission limits |
|  |  | 溢出或释放（如废气、废水排放）触发报告要求并逾越出了数据中心运营区域的属地 Spills or releases (e.g., exhaust gas, wastewater) triggering reporting requirements and extending beyond the jurisdiction of the data center's operational area | 溢出或释放（如废气、废水排放）触发报告要求但仍在数据中心运营属地 Spills or releases (e.g., exhaust gas, wastewater) triggering reporting requirements but remaining within the jurisdiction of the data center's operational area | 溢出或释放（如废气、废水排放）未触发报告限值 Spills or releases (e.g., exhaust gas, wastewater) not triggering reporting limits |
|  |  | 外部发现的违反废弃物处置或危险品运输法规规定 Violations of waste disposal or hazardous materials transportation regulations discovered externally | 数据中心自我识别的违规，废弃物处理或危险品运输法规 Data center self-identified violations of waste disposal or hazardous materials transportation regulations |  |
| 火灾/爆炸 Fire/Explosion | 火灾或爆炸影响社区或环境，越过围墙，中断公共服务或触发社区保护措施。 Fire or explosion affecting the community or environment, crossing the perimeter wall, disrupting public services, or triggering community protection measures | 火灾或爆炸造成严重损害，损失超过100万元。 Fire or explosion causing significant damage, with losses exceeding 1 million yuan | 火灾或爆炸需要外部应急响应者的协助或引起损失超过50万元。 Fire or explosion requiring assistance from external emergency responders or causing losses exceeding 500,000 yuan | 火灾爆炸被扑灭内部响应者并导致损失少于10万元。 Fire or explosion extinguished by internal responders, resulting in losses of less than 100,000 yuan |

## 6.3 EHS事故事件报告 EHS incident report

6.3.1生产安全事故发生后，事故现场有关人员应当立即向数据中心主要负责人报告；情况紧急时，事故现场有关人员可以直接向事故发生地县级以上人民政府安全生产监督管理部门和负有安全生产监督管理职责的有关部门报告。

After a production safety accident occurs, relevant personnel at the accident scene shall immediately report to the main responsible person of the data center. In emergency situations, relevant personnel at the accident scene may directly report to the work safety supervision and administration department of the local government at or above the county level where the accident occurred, as well as to other relevant departments responsible for work safety supervision and administration.

6.3.2数据中心主要负责人接到报告后，应当于1小时内向事故发生地县级以上人民政府安全生产监督管理部门和负有安全生产监督管理职责的有关部门报告。

Upon receiving the report, the main responsible person of the data center shall, within one hour, report to the work safety supervision and administration department of the local government at or above the county level where the accident occurred, as well as to other relevant departments responsible for work safety supervision and administration.

6.3.3向当地县级以上人民政府安全生产监督管理部门和负有安全生产监督管理职责的有关部门报告时，报告事故应当包括下列内容：

When reporting to the work safety supervision and administration department of the local government at or above the county level and other relevant departments responsible for work safety supervision and administration, the accident report shall include the following information:

- 事故发生单位概况；

An overview of the unit where the accident occurred.

- 事故发生的时间、地点以及事故现场情况；

The time and location of the accident, as well as the situation at the accident scene.

- 事故的简要经过；

A brief description of the accident.

- 事故已经造成或者可能造成的伤亡人数（包括下落不明的人数）和初步估计的直接经济损失；

The number of casualties (including missing persons) caused or potentially caused by the accident, as well as the preliminary estimate of direct economic losses.

- 已经采取的措施；

Measures that have already been taken.

- 其他应当报告的情况；

Other information that should be reported.

- 使用电话快报，应当包括下列内容；

When using a telephone express report, the following information shall be included:

  - 事故发生单位的名称、地址、性质；

The name, address, and nature of the unit where the accident occurred.

  - 事故发生的时间、地点；

The time and location of the accident.

  - 事故已经造成或者可能造成的伤亡人数（包括下落不明、涉险的人数）。

The number of casualties (including missing and endangered persons) caused or potentially caused by the accident.

6.3.4事故报告后出现新情况的，应当及时补报。自事故发生之日起30日内，事故造成的伤亡人数发生变化的，应当及时补报。道路交通事故、火灾事故自发生之日起7日内，事故造成的伤亡人数发生变化的，应当及时补报。

If new circumstances arise after the accident report is submitted, a supplementary report shall be submitted promptly. If the number of casualties changes within 30 days from the date of the accident, a supplementary report shall be submitted promptly. For road traffic accidents and fire accidents, if the number of casualties changes within 7 days from the date of the accident, a supplementary report shall be submitted promptly.

6.3.5发生工伤事故、交通事故等，员工、承包商和分包商应按照EHS事故汇报流程立即向相应主管、经理或者EHS管理负责人报告，现场人员需要立即开展应急处置。

In the event of work-related injuries, traffic accidents, or similar incidents, employees, contractors, and subcontractors shall immediately report to the relevant supervisor, manager, or EHS officer in accordance with the EHS incident reporting procedures. Personnel on-site shall initiate emergency response measures immediately.

6.3.6在万国数据内部，数据中心员工受伤上报流程为：

Within GDS, the reporting process for data center employee injuries is as follows:

- 员工工作受伤后，受伤员工应立即停止工作；

After an employee is injured at work, the injured employee shall immediately stop working.

- 寻求现场紧急急救处置；

Seek immediate first aid treatment on-site.

- 现场受伤员工或现场其他人员报送数据中心经理；

The injured employee or other personnel on-site shall report the incident to the data center manager.

- 数据中心经理接到信息10分钟内需上报至片区负责人；

The data center manager shall report to the district responsible person within 10 minutes of receiving the information.

- 片区负责人需30分钟内电话及1小时内书面报送至运维EHS；

The district responsible person shall report to the operations EHS by phone within 30 minutes and in writing within 1 hour.

- 运维EHS将信息上报至GDS。

The operation EHS shall report the information to GDS.

6.3.7环境相关事故事件以及职业病危害相关事故事件的管理参考相关管理制度。

The management of environment-related incidents and occupational health hazard-related incidents shall refer to the relevant management procedures.

6.3.8数据中心事故报告信息需填报事故报告表，详见附件1。

The accident report information of the data center shall be recorded in the accident report form. For details, refer to Attachment 1.

## 6.4 EHS事故事件的应急响应行动 Emergency response actions for EHS incidents

6.4.1现场应按照EHS事故事件情况启动适用于事故级别的应急响应计划和相关程序。事故发生单位负责人接到事故报告后，应当立即启动相应的事故应急预案，采取有效措施，积极组织抢救，并对本单位各类重大危险源实施有效监控，防止事故扩大或者引发次生事故，最大限度地减少人员伤亡和财产损失。

On-site personnel shall activate the emergency response plan and relevant procedures appropriate to the level of the EHS incident. Upon receiving the accident report, the person in charge of the unit where the accident occurred shall immediately initiate the corresponding emergency response plan, take effective measures, actively organize rescue efforts, and effectively monitor all major hazard sources within the unit to prevent the accident from escalating or triggering secondary incidents, thereby minimizing casualties and property losses.

6.4.2发生EHS事故事件后，现场应为受伤或生病员工提供充分的便利以获得及时的医疗救治。

After an EHS incident occurs, the site shall provide adequate facilities to ensure that injured or ill employees receive timely medical treatment.

- 对于轻微受伤，员工可以自行实施急救处理或由受过急救培训的人员照看。

For minor injuries, employees may administer first aid themselves or be assisted by personnel trained in first aid.

- 对于严重和重大伤害/疾病，现场应提供充分便利，以获得在现场诊所/附近医疗机构的及时医疗救治。

For severe and major injuries/illnesses, the site shall provide adequate facilities to ensure timely medical treatment at the on-site clinic or nearby medical institutions.

6.4.3发生工伤事故后，需要按照《工伤事故管理制度》要求进行管理。

After a work-related injury accident occurs, it shall be managed in accordance with the "Work-related Injury Accident Management System."

## 6.5 EHS事故事件调查和报告 EHS incidents investigation and report

6.5.1现场EHS管理负责人或相关人员应确认事故现场是否受到适当的保护，包括收集和保存任何额外的证据。

The on-site EHS officer or relevant personnel shall ensure that the accident scene is properly secured, including collecting and preserving any additional evidence.

6.5.2必须对所有EHS事故事件进行调查，调查的深度取决于事件的严重程度或潜在的重大后果。在确定事故现场后，应立即开始调查。

All EHS incidents must be investigated. The depth of the investigation shall depend on the severity of the incident or its potential major consequences. The investigation shall commence immediately after securing the accident scene.

6.5.3成立EHS事故事件调查小组，小组成员包括事故发生部门现场人员、主管、经理以及EHS管理负责人、风控部门（如需要）等。对于重大和严重事件，调查负责人应独立于与事故有直接关系的团队。参加事故调查的人员需要具备事故调查技术知识。

An EHS incident investigation team shall be established, consisting of on-site personnel, supervisors, and managers from the department where the accident occurred, the EHS officer, and the risk control department (if required). For significant and serious incidents, the investigation leader shall be independent of the teams directly involved in the incident. Investigators must possess technical knowledge of accident investigation.

6.5.4根据事故的性质，调查可以使用不同的方法。调查负责人与团队一起应确定最佳方法或适当的方法组合。事故事件调查包括但不限于：鱼骨图、5Why等。

Depending on the nature of the accident, various investigation methods may be used. The investigation leader and the team shall determine the most suitable method or combination of methods. Accident investigation methods include, but are not limited to, fishbone diagrams and the 5 Whys analysis.

6.5.5事故发生部门负责编写事故调查报告，报告中主要包括事故发生的时间、地点、人员伤害、事故过程描述、已采取的应急处置措施、事故根本原因分析、整改措施等。事故调查报告需要在事故发生后的两周内完成。

The department where the accident occurred is responsible for preparing the accident investigation report. The report shall include the time and location of the accident, personnel injuries, a description of the accident process, emergency response measures taken, root cause analysis, and corrective actions. The report shall be completed within two weeks of the accident.

6.5.6事故报告信息需要及时进行统计归档处理。

The accident report information shall be statistically archived in a timely manner.

6.5.7 事故调查报告模板可参考附件2。

The accident investigation report template can be found in Attachment 2.

## 6.6 EHS事故事件学习和审查 EHS incidents learning and review

6.6.1在发生较大、严重、重大事故发生后，事故发生部门负责人应定期组织会议，回顾和验证事故调查的根本原因，追踪事故整改措施的执行情况，参加会议人员主要包括事故发生部门主管、经理、EHS管理负责人、风控部门人员等相关人员。

After medium-level, serious, or major accidents occur, the department responsible person where the accident occurred shall organize regular meetings to review and validate the root causes identified in the accident investigation and track the implementation of corrective actions. Attendees shall primarily include supervisors, managers, the EHS officer, and risk control personnel from the department where the accident occurred, as well as other relevant personnel.

6.6.2在收到调查结论后，现场应将事故事件调查中的相关学习经验传达给受影响的人的员工和承包商/分包商。

Upon receiving the investigation conclusions, the site shall communicate the lessons learned from the incident investigation to affected employees, contractors, and subcontractors.

6.6.3各部门/岗位/业务线可针对事故学习开展相关人员培训。汲取的教训应用于更新程序/作业指导书、风险评估、培训或其他适用工具。所学到的经验教训应通过数据中心内部会议、公告栏公示或其他方法传达给员工。

All departments, positions, and business lines may conduct relevant training based on lessons learned from accidents. The lessons learned shall be used to update procedures, work instructions, risk assessments, training materials, and other applicable tools. These lessons shall be communicated to employees through internal meetings, bulletin boards, or other means within the data center.

## 6.7 OSHA可记录事件管理 OSHA recordable incident management

6.7.1 OSHA（Occupational Safety and Health Administration，美国职业安全与健康管理局）要求记录死亡、伤害和疾病记录的每个雇主（上海浦江数据中心）必须记录每起死亡、伤害和疾病：

OSHA (Occupational Safety and Health Administration) requires every employer (SHANGHAI PUJIANG Data Center) that records deaths, injuries, and illnesses to document each death, injury, and illness:

- 与工作有关；

Work-related.

- 是新病例；

A new case.

- 符合一项或多项一般记录标准或适用于其他的一些特定情况；

Meets one or more general recording criteria or applies to certain specific situations.

6.7.2如果工作环境中的事件或暴露导致或促成了由此产生的状况，或显著加重了先前存在的伤害或疾病，则您必须将伤害或疾病视为与工作有关。除非5.6.3、5.6.4中的例外情况特别适用，否则因工作环境中发生的事件或暴露而导致的伤害和疾病被推定为与工作有关。

If an incident or exposure in the work environment caused or contributed to the resulting condition or significantly aggravated a pre-existing injury or illness, it must be regarded as work-related. Injuries and illnesses resulting from incidents or exposures in the work environment are presumed to be work-related unless the exceptions specified in sections 5.6.3 and 5.6.4 apply.

6.7.3在工作环境中发生的伤害或疾病属于以下例外情况之一，与工作无关，因此不可记录：

Injuries or illnesses occurring in the work environment that fall under one of the following exceptions are not work-related and therefore not recordable.

表2 不可记录的事件

Table 2: Non-Recordable Incidents

| 1904.5（b）（2） | 如果出现以下情况，您无需记录受伤和疾病： You do not need to record injuries and illnesses if the following situations occur: |  |
| --- | --- | --- |
| （I） | 在受伤或生病时，雇员（员工）是作为公众的一员而不是作为雇员出现在工作环境中。 When injured or ill, the employee was in the work environment as a member of the general public rather than as an employee. |  |
| (II) | 伤害或疾病涉及在工作中出现的体征或症状，但仅由与工作无关的事件或在工作环境之外发生的暴露引起。 The injury or illness involves signs or symptoms that surface at work but are solely caused by non-work-related incidents or exposures occurring outside the work environment. |  |
| (III) | 受伤或疾病完全是由于自愿参加健康计划或医疗、健身或娱乐活动（如献血、体格检查、流感疫苗、运动课、壁球或棒球）造成的。 The injury or illness is entirely the result of voluntary participation in wellness programs, medical, fitness, or recreational activities (e.g., blood donation, physical examinations, flu shots, exercise classes, squash, or baseball). |  |
| (IV) | 受伤或疾病完全是由于雇员进食、饮水或准备供个人消费的食物或饮料（无论是在雇主处所购买还是带入）造成的。例如，如果雇员在雇主的场所内因被三明治噎住而受伤，则该案件将不被视为与工作有关。 注意：如果雇员因摄入被工作场所污染物（例如铅）污染的食物而生病，或因雇主提供的食物而食物中毒，则该个案将被视为与工作有关。 The injury or illness is entirely caused by the employee eating, drinking, or preparing food or beverages for personal consumption (whether purchased at the employer’s premises or brought in). For example, if an employee is injured by choking on a sandwich at the employer’s premises, the case will not be considered work-related. Note: If an employee becomes ill from consuming food contaminated by workplace pollutants (e.g., lead) or suffers food poisoning from food provided by the employer, the case will be considered work. |  |

续表2 不可记录的事件

Continued Table 2: Non-Recordable Incidents

| 1904.5（b）（2） | 如果出现以下情况，您无需记录受伤和疾病： You do not need to record injuries and illnesses if the following situations occur: |
| --- | --- |
| (V) | 受伤或疾病完全是由于雇员在指定的工作时间之外在机构从事个人任务（与其工作无关）造成的。 The injury or illness is entirely the result of the employee performing personal tasks (unrelated to work) at the workplace outside designated working hours. |
| (VI) | 受伤或疾病完全是个人修饰、针对与工作无关的疾病进行自我治疗的结果，或者是故意造成的。 The injury or illness is entirely due to personal grooming, self-treatment for a non-work-related illness, or was intentionally self-inflicted. |
| (VII) | 伤害或疾病是由机动车事故引起的，发生在员工上下班时去往公司停车场的公司通道上。 The injury or illness resulted from a motor vehicle accident on the company’s access road while the employee was commuting. |
| (VIII) | 该疾病是普通感冒或流感（注意：如果员工在工作中被感染，则肺结核、布鲁氏菌病、甲型肝炎或鼠疫等传染病被视为与工作有关）。 The illness is a common cold or flu. (Note: Infectious diseases such as tuberculosis, brucellosis, hepatitis A, or plague are considered work-related if the employee is infected at work.) |
| (IX) | 这种疾病是一种精神疾病。除非雇员自愿向雇主提供医生或其他受过适当培训和经验的持证医疗保健专业人员（精神科医生、心理学家、精神科执业护士等）的意见，说明雇员患有与工作有关的精神疾病，否则精神疾病将不被视为与工作有关。 The illness is a mental illness. Mental illnesses are not considered work-related unless the employee voluntarily provides the employer with an opinion from a physician or other licensed healthcare professional (e.g., psychiatrists, psychologists, psychiatric nurse practitioners, etc.) with appropriate training and experience, stating that the employee has a work-related mental illness. |

6.7.4如果员工符合下列例外情况之一，则不必记录员工处于旅行状态时发生的伤害或疾病：

If an employee meets one of the following exceptions, injuries or illnesses occurring while the employee is traveling do not need to be recorded.

表3 不可记录的事件-员工处于旅行状态时

Table 3 - Non-recordable Incidents - When Employees Are in a Travel Status

| 1904.5（b）（6） | 如果员工有 . . . If an employee has... | 您可以使用以下方法来确定伤害或疾病是否与工作有关： You can use the following methods to determine if an injury or illness is work-related: |
| --- | --- | --- |
| (I) | 入住酒店或汽车旅馆一天或多天 Checked into a hotel or motel for one or more days. | 当出差员工入住酒店、汽车旅馆或其他临时住所时，他或她建立了一个“家外之家”。您必须在员工入住酒店、汽车旅馆或其他临时住所后评估其与工作相关的活动，就像评估非旅行员工的活动一样。当员工入住临时住所时，他或她被视为已离开工作环境。当员工每天开始工作时，他或她会重新进入工作环境。如果员工已经建立了一个“家外之家”，并且每天到固定的工作场所报到，那么如果员工在临时住所和工作地点之间通勤时发生的伤害或疾病，您也不会认为这些伤害或疾病与工作有关。 When a traveling employee checks into a hotel, motel, or other temporary accommodation, they establish a "home away from home." You must evaluate their work-related activities after check-in as you would for non-traveling employees. Once checked in, they are considered to have left the work environment and re-enter it when starting work each day. If they have established a "home away from home" and report to a fixed workplace daily, any injuries or illnesses occurring during their commute between the temporary accommodation and the workplace are not considered work-related. |
| (II) | 因个人原因绕道而行 Take a detour for personal reasons. | 如果伤害或疾病发生在员工从合理直接的旅行路线绕道而行时（例如，出于个人原因进行了一次旁路旅行，改变了正常路径），则不被视为与工作有关。 If an injury or illness occurs when an employee deviates from a reasonably direct travel route (e.g., takes a side trip for personal reasons, altering the normal path), it is not considered work-related. |

6.7.5 OSHA将工作环境定义为“一名或多名员工正在工作或作为其就业条件在场的场所和其他地点。工作环境不仅包括物理位置，还包括员工在工作过程中使用的设备或材料。

OSHA defines the work environment as "the establishment and other locations where one or more employees are working or are present as a condition of their employment. The work environment includes not only physical locations but also the equipment or materials used by employees in the course of their work.

6.7.6如果受伤或疾病导致以下任何一种情况，您必须考虑符合一般记录标准，因此可以记录：

If an injury or illness results in any of the following, you must consider that the general recording criteria are met, and it is recordable:

- 死亡

Death

- 损失工时

Lost work time

- 工作受限或转移到另一份工作

Restricted work or transfer to another job

- 急救以外的医疗或失去知觉（意识丧失）

Medical treatment other than first aid or loss of consciousness

如果事故事件涉及医生或其他有执照的医疗保健专业人员诊断出的重大伤害或疾病，即使它不会导致死亡、缺勤天数、工作受限或工作调动、急救以外的医疗或失去知觉，也必须考虑符合一般记录标准。

If an incident involves a significant injury or illness diagnosed by a physician or other licensed healthcare professional, the general recording criteria must be considered met, even if it does not result in death, days away from work, restricted work, job transfer, medical treatment beyond first aid, or loss of consciousness.

6.7.7如何确定受伤或疾病是否导致工作受限？当因工伤或疾病导致以下情况时，就会发生限制工作：

How to determine if an injury or illness results in work restrictions? Work restrictions occur when the following situations are caused by a work-related injury or illness:

- 阻止员工执行其工作的一项或多项日常职能（出于记录保存的目的，员工的日常职能是员工每周至少定期执行一次的工作活动。），或阻止他或她本应安排工作的整个工作日工作；

Prevents the employee from performing one or more routine job functions (for record-keeping purposes, an employee's routine functions are work activities performed regularly at least once a week) or prevents the employee from working an entire scheduled workday.

- 医生或其他有执照的医疗保健专业人员建议员工不要执行其工作的一项或多项日常职能，或者不要在他或她本来应该工作的整个工作日工作；

A physician or other licensed healthcare professional advises the employee not to perform one or more routine job functions or not to work an entire workday as scheduled.

6.7.8 “医疗处理”是指对患者进行管理和护理以对抗疾病或紊乱。医疗处理不包括：

"Medical treatment" refers to the management and care of a patient to combat a disease or disorder. It does not include:

- 仅出于观察或咨询目的而去看医生或其他有执照的医疗保健专业人员；

Visiting a physician or other licensed healthcare professional solely for observation or consultation.

- 进行诊断程序，例如 X 光检查和血液检查，包括使用仅用于诊断目的的处方药（例如，散瞳眼药水） ；

Diagnostic procedures such as X-rays and blood tests, including the use of prescription medications solely for diagnostic purposes (e.g., dilating eye drops).

- “急救”

First aid.

6.7.9“急救”是指以下内容 "First aid" refers to the following:

- 使用非处方强度的非处方药物（对于处方和非处方形式的药物，医生或其他有执照的医疗保健专业人员建议使用处方强度的非处方药物被视为用于记录保存目的的医疗）；

Using over-the-counter medications at non-prescription strength. (For medications available in both prescription and over-the-counter forms, a recommendation by a physician or other licensed healthcare professional to use over-the-counter medications at prescription strength is considered medical treatment for record-keeping purposes.)

- 进行破伤风免疫接种（其他免疫接种，如乙型肝炎疫苗或狂犬病疫苗，被视为医疗治疗）；

Tetanus immunizations. (Other immunizations, such as hepatitis B or rabies vaccines, are considered medical treatment.)

- 清洁，冲洗或浸泡皮肤表面的伤口；

Cleaning, flushing, or soaking wounds on the skin surface.

- 使用诸如绷带、创可贴™、纱布垫或蝶形绷带、SteriStrips™等伤口覆盖物（其他伤口闭合装置如缝线、钉书钉被视为医疗治疗）；

Using wound coverings such as bandages, Band-Aids™, gauze pads, butterfly bandages, or SteriStrips™. (Other wound closure devices, such as sutures or staples, are considered medical treatment.)

- 使用热疗法或冷疗法；

Using heat or cold therapy.

- 使用任何非刚性支撑手段，例如弹性绷带、包裹、非刚性背带等（带有刚性支撑的装置或其他旨在固定身体部位的系统被视为用于记录保存目的的医疗）；

Using non-rigid supports such as elastic bandages, wraps, or non-rigid braces. (Devices with rigid supports or other systems designed to immobilize body parts are considered medical treatment for record-keeping purposes.)

- 在运送事故受害者时使用临时固定装置（例如夹板、吊带、颈圈、背板等）；

Using temporary immobilization devices such as splints, slings, neck collars, or backboards when transporting accident victims.

- 钻指甲或脚趾甲以减轻压力，或从水疱中排出液体；

Drilling a fingernail or toenail to relieve pressure or draining fluid from a blister.

- 使用眼罩；

Using an eye patch.

- 仅使用冲洗或棉签清除眼睛中的异物；

Using irrigation or cotton swabs to remove foreign objects from the eye.

- 通过冲洗、镊子、棉签或其他简单方法清除眼睛以外区域的碎片或异物；

Removing debris or foreign objects (other than from the eyes) by irrigation, tweezers, cotton swabs, or other simple methods.

- 使用手指保护装置；

Using finger guards.

- 使用按摩（物理治疗或脊椎按摩疗法被视为用于记录保存目的的医疗）；

Using massage. (Physical therapy or chiropractic care is considered medical treatment for record-keeping purposes.)

- 饮用液体以缓解热应激；

Drinking fluids to relieve heat stress.

## 6.8 沟通与培训Communication and training

6.8.1将本管理制度中规定的EHS事故事件管理相关的各部门职责以及要求告知相关人员，包括在数据中心内有作业活动的外部承包商。

Inform relevant personnel, including external contractors conducting operational activities within the data center, of the responsibilities and requirements for EHS incident and accident management as stipulated in this management system.

6.8.2将EHS事故事件管理列入到年度培训计划中，按照计划对相关人员开展培训，主要包括事故事件分级、事故事件汇报流程、应急处置、事故事件调查等，参加培训人员主要包括EHS管理负责人，各部门/岗位/业务线人员等。

Include EHS incident management in the annual training plan and conduct training for relevant personnel as scheduled. The training shall cover incident classification, reporting procedures, emergency response, and incident investigation. Trainees shall primarily include the EHS officer and personnel from all departments, positions, and business lines.

6.8.3负责领导事故事件调查的人员，应接受开展调查和根本原因分析的培训（例如 5 why分析法，鱼骨图分析法等）。

Personnel responsible for leading accident and incident investigations shall receive training in investigation techniques and root cause analysis (e.g., 5 Whys analysis, fishbone diagram analysis).

6.8.4事故事件发生后，需要按照规定时间将事故事件进行记录，也包括承包商在数据中心内开展作业活动发生的事故。

After accidents or incidents occur, they shall be recorded within the specified time frame, including incidents involving contractors' operational activities within the data center.

6.8.5员工培训要求应在事故事件调查后进行适当地回顾和更新。

Employee training requirements shall be appropriately reviewed and updated following accident investigations.

6.8.6事故发生后，事故现场有关人员应当立即向数据中心主要负责人报告；数据中心主要负责人接到报告后，应当结合现场实际情况，决定是否需要于1小时内向事故发生地县级以上人民政府安全生产监督管理部门和负有安全生产监督管理职责的有关部门报告。情况紧急时，事故现场有关人员可以直接向事故发生地县级以上人民政府安全生产监督管理部门和负有安全生产监督管理职责的有关部门报告。

After an accident occurs, relevant personnel at the scene shall immediately report to the head of the data center. Upon receiving the report, the head shall, based on the actual situation, decide whether to report to the work safety supervision and administration department of the local government at or above the county level and other relevant departments responsible for work safety supervision and administration within one hour. In emergency situations, personnel at the scene may directly report to the aforementioned departments.

6.8.7发生人员伤亡事故后，数据中心应立即开展事故应急处置，并配合政府相关部门进行事故调查以及后续事故调查报告等事项。

After a casualty accident occurs, the data center shall immediately initiate emergency response measures and cooperate with relevant government departments in the accident investigation and subsequent reporting processes.

7 附件Attachments

- IF-PJ-EHS-2101：事故报告表 Accident Report Form

- IF-PJ-EHS-2102：事故调查报告 Accident Investigation Report