库房EHS管理制度

Warehouse EHS Management System

内部使用

在使用前，请和文档编写人员确认本文档为最新版本。

未经上海暻熠数据科技有限公司、上海暻耀网络技术有限公司书面许可，其它任何机构或个人不可传阅、引用或复制本文件的内容。

Internal use

Before using, check with the documentation writer to make sure this document is the latest version.

Without the written permission of Shanghai Jingyi Data Technology Co., Ltd., Shanghai Jingyao internet Technology Co., Ltd., no other organization or individual is allowed to circulate, quote or copy the content of this document.

| 文件编号Documents | IP-PJ-EHS-23 |
| --- | --- |
| 版 本Edition | V1.0 |
| 文件维护部门File maintenance department | 上海浦江数据中心 |
| 会签部门Document Signing department | 上海浦江数据中心 |
| 执行范围Scope of execution | 上海浦江数据中心 |

文件修订记录

Document revision record

| 版本号 Edition | 版本日期 Date | 修订内容 Revised Content | 修订人员 Revision personnel | 核准员 Approver | 备注 Remarks |
| --- | --- | --- | --- | --- | --- |
| V1.0 | 2025-04-16 | 首次修订 | 楚靖岳 | 陆磊 |  |

目录Content

1 目的Purpose	4

2 适用范围Scope of application	4

3 规范性引用文件Reference documents	4

4 术语和定义Terms and definitions	4

5 职责Responsibility	5

5.1 各部门/岗位/业务线All relevant departments/positions/business lines	5

5.2 EHS管理负责人EHS officer	5

6 程序program	6

6.1 库房的检查Inspection of the warehouse	6

6.2 库房EHS管理要求EHS Management Requirements for Warehouse	6

6.3 库房消防设施要求Requirements for fire protection facilities in warehouses	9

6.4 应急与演练Emergency and drill	10

7 附件Attachments	10

1 目的Purpose

本制度旨在建立上海暻熠数据科技有限公司、上海暻耀网络技术有限公司（以下简称“上海浦江数据中心”）安全生产责任制度。目标包括确保遵守国家和地方法律法规、国外相关法规、客户要求以及上海浦江数据中心内部要求，明确并落实各岗位人员的安全生产责任，预防和减少生产安全事故的发生，降低运营EHS风险，并促进持续改进。

This system aims to establish the EHS management process for the warehouse of Shanghai Jingyi Data Technology Co., Ltd., Shanghai Jingyao internet Technology Co., Ltd.. (hereinafter referred to as "SHANGHAI Pujiang Data Center"). The objectives include ensuring compliance with national and local laws and regulations, relevant foreign regulations, customer requirements, and internal requirements of SHANGHAI PUJIANG Data Center, ensuring warehouse EHS management compliance, reducing operational EHS risks, and promoting continuous improvement.

2 适用范围scope of application

适用于上海浦江数据中心，以及所有与EHS有关事项的管理职能部门。

Applicable to SHANGHAI PUJIANG Data Center and all management functional departments related to EHS matters.

3 规范性引用文件Normative reference documents

《中华人民共和国安全生产法》（2021年修订）

Work Safety Law of the People's Republic of China (Revised in 2021)

《中华人民共和国消防法》（2021年修订）

Fire Protection Law of the People's Republic of China (Revised in 2021)

《生产安全事故应急预案管理办法》（2019年修订）

Management Measures for Emergency Plans for Production Safety Accidents (Revised in 2019)

《仓库防火安全管理规则》（中华人民共和国公安部令第6号）

Warehouse Fire Safety Management Rules (Order No. 6 of the Ministry of Public Security of the People's Republic of China)

GB 50016   建筑防火设计规范

GB 50016 Code for Fire Protection Design of Buildings

GB 50140   建筑灭火器配置设计规范

GB 50140 Code for Design of Fire Extinguisher Configuration in Buildings

GA 1131    仓储场所消防安全管理通则

GA 1131 General Rules for Fire Safety Management of Warehousing Sites

4 术语和定义Term and definition

无

not have

5 职责responsibility

5.1 各部门/岗位/业务线Each department/position/business line

5.1.1各部门/部门/业务应遵守上海浦江数据中心库房EHS管理规定。

Each department/business should comply with the EHS management regulations for the SHANGHAI PUJIANG Data Center warehouse.

5.1.2掌握上海浦江数据中心库房消防器材的操作使用。

Master the operation and use of fire-fighting equipment in the warehouse of SHANGHAI PUJIANG Data Center.

5.1.3掌握库房初期火灾的扑救方法，参与应急演练。

Master the firefighting methods for initial warehouse fires and participate in emergency drills.

5.2 EHS管理负责人EHS officer

5.2.1负责组织制定上海浦江数据中心库房EHS管理制度和消防安全操作规程。

Responsible for organizing the development of EHS management system and fire safety operation procedures for the warehouse of SHANGHAI PUJIANG Data Center.

5.2.2组织对库房消防设施、灭火器材和消防安全标志的维护保养，保障消防通道、安全出口和消防车通道畅通。

Organize the maintenance and upkeep of fire protection facilities, firefighting equipment, and fire safety signs in the warehouse, ensuring smooth access to fire exits, safety exits, and fire truck passages.

5.2.3组织开展上海浦江数据中心库房防火检查，消除火灾隐患。

Organize a fire prevention inspection of the SHANGHAI PUJIANG Data Center warehouse to eliminate fire hazards.

5.2.4组织上海浦江数据中心库房相关员工的消防知识、技能的教育和培训。

Organize education and training on fire safety knowledge and skills for employees related to the warehouse of SHANGHAI PUJIANG Data Center.

5.2.5组织上海浦江数据中心库房灭火和应急预案的实施和消防演练。

Organize the implementation of fire extinguishing and emergency plans, as well as fire drills, for the warehouse of SHANGHAI PUJIANG Data Center.

6 程序program

6.1 库房的检查Inspection of the warehouse

6.1.1EHS管理负责人每月组织一次防火检查，各部门/岗位/业务线每周对库房进行一次防火检查。

The EHS officer shall organize a fire prevention inspection once a month, and each department/position/business line shall conduct a fire prevention inspection of the warehouse once a week.

6.1.2防火检查内容包括以下内容：

The fire prevention inspection includes the following contents:

- 库房消防安全制度和消防安全操作规程执行与落实情况；

Implementation and execution of warehouse fire safety regulations and fire safety operation procedures.

- 库房发现的隐患整改落实情况；

Implementation status of rectification of hidden dangers discovered in the warehouse.

- 库房所属部门管理人是否掌握消防知识；

Has the manager of the department to which the warehouse belongs mastered fire safety knowledge.

- 库房是否设置办公室及员工宿舍；

Is the warehouse equipped with offices and employee dormitories.

- 储存物品是否分类、分组和分堆（垛）存放，防火间距是否满足要求，是否存放影响消防安全的物品等；

Whether the stored items are classified, grouped, and stacked, whether the fire separation distance meets the requirements, and whether items that affect fire safety are stored.

- 火源、电源管理情况，用火、用电有无违章；

Management of fire sources and power sources, and whether there are any violations of fire and electricity usage regulations.

- 消防通道、安全出口、消防车通道是否畅通，是否有明显的安全标志；

Whether the fire exits, safety exits, and fire truck passages are unobstructed and have clear safety signs.

- 消防水源情况，灭火器配置及完好情况，消防设施有无损坏、停用、埋压、遮挡、圈占等使用情况。

The condition of the fire water source, the configuration and integrity of fire extinguishers, and whether the fire protection facilities are damaged, out of service, buried, obstructed, or occupied.

6.2 库房EHS管理要求EHS Management Requirements for Warehouse

6.2.1上海浦江数据中心库房不应设置员工宿舍。

The warehouse of SHANGHAI PUJIANG Data Center should not have employee dormitories.

6.2.2物品入库前应有专人负责检查，确认无火种等隐患后，方可入库。

Before the items are stored, a dedicated person should be responsible for inspecting and confirming that there are no hidden dangers such as fire before they can be stored.

6.2.3库房储存物品应分类、分堆、限额存放。每个堆垛的面积不应大于150m2。

Items stored in the warehouse should be classified, stacked, and stored within limits. The area of each stack should not exceed 150m2.

6.2.4库房内主通道的宽度不应小于2m。

The width of the main passage in the warehouse should not be less than 2 meters.

6.2.5库房内堆放物品时应满足以下要求：

When stacking items in the warehouse, the following requirements should be met:

堆垛上部与楼板、平顶屋之间的距离不小于0.3m(如果为人字屋顶从横梁算起)；

The distance between the upper part of the stack and the floor slab or flat roof shall not be less than 0.3m (calculated from the crossbeam if it is a herringbone roof).

- 物品与照明灯之间的距离不小于0.5m；

The distance between the item and the lighting fixture shall not be less than 0.5m.

- 物品与墙之间的距离不小于0.5m；

The distance between the item and the wall shall not be less than 0.5m.

- 物品堆垛与柱之间的距离不小于0.3m；

The distance between the stack of items and the column shall not be less than 0.3m.

- 物品堆垛与堆垛之间的距离不小于1m。

The distance between stacking items shall not be less than 1m.

6.2.6储存的物品与风管、供暖通道、散热器的距离不应小于0.5m，与供暖机组、风管炉、烟道之间的距离在各个方向上都不应小于1m。

The distance between stored items and air ducts, heating channels, and radiators should not be less than 0.5m, and the distance between stored items and heating units, air ducts, furnaces, and flues should not be less than 1m in all directions.

6.2.7库房内需要设置货架堆放物品时，货架应采用非燃烧材料制作。货架不应遮挡消火栓、自动喷淋系统以及排烟口。

When it is necessary to set up shelves for storing items in the warehouse, the shelves should be made of non-combustible materials. Shelves should not obstruct fire hydrants, automatic sprinkler systems, and exhaust vents.

6.2.8库房内的电器设备应与可燃物保持不小于0.5m的防火间距，架空线路的下方不应堆放物品。

Electrical equipment in the warehouse should maintain a fire separation distance of no less than 0.5m from combustible materials, and no items should be stacked below overhead power lines.

6.2.9每个库房应在库房外单独安装电器开关箱，各部门/岗位/业务线离开库房时，应切断场所的非必要电源。

Each warehouse should have a separate electrical switch box installed outside the warehouse. When departments/positions/business lines leave the warehouse, non-essential power sources in the area should be cut off.

6.2.10库房内所敷设的配电线路，应穿金属管或难燃硬塑料管保护。不应随意乱接电线，擅自增加用电设备。

The distribution lines laid in the warehouse should be protected by metal pipes or flame-retardant hard plastic pipes. Do not randomly connect wires or add electrical equipment without authorization.

6.2.11库房内不应使用电路、电烙铁、电熨斗、电热水器等电热器具和电视机、电冰箱等家用电器。

Electric circuits, soldering irons, irons, water heaters, and household appliances such as televisions and refrigerators should not be used in the warehouse.

6.2.12库房的电气线路、电气设备应定期检查、检测，禁止长时间超负荷运行。

The electrical circuits and equipment in the warehouse should be regularly inspected and tested, and long-term overload operation is prohibited.

6.2.13库房内禁止吸烟，并在醒目处设置“禁止吸烟”的标志。

Smoking is prohibited in the warehouse, and a "No Smoking" sign should be prominently displayed.

6.2.14库房的电气设备应设专人管理，由持证的电工进行安装和维修。发现漏电、老化、绝缘不良、接头松动、电线互相缠绕等可能引起打火。短路、发热时，应立即停止使用，并及时修理或更换。禁止带电移动电气设备或接线、检修。

The electrical equipment in the warehouse should be managed by a dedicated person and installed and maintained by certified electricians. Discovering leakage, aging, poor insulation, loose joints, and tangled wires may cause ignition. When there is a short circuit or heating, it should be stopped immediately and repaired or replaced in a timely manner. It is prohibited to move live electrical equipment or conduct wiring and maintenance.

6.2.15上海浦江数据中心库房内不应使用明火，并应设置醒目的禁止标志。因施工确需明火作业时，应按用火管理制度办理动火证，由具有相应资格的专门人员进行动火操作，并设专人和灭火器材进行现场监护；动火作业结束后，应检查并确认无遗留火种。动火证应注明动火地点、时间、动火人、现场监护、批准人和防火措施等内容。

Open flames should not be used in the warehouse of SHANGHAI PUJIANG Data Center, and conspicuous prohibition signs should be set up. When it is necessary to carry out open flame operations during construction, a hot work permit should be obtained in accordance with the fire management system, and specialized personnel with corresponding qualifications should carry out hot work operations. A dedicated person and firefighting equipment should be assigned to supervise the site; After the hot work is completed, it should be checked and confirmed that there are no remaining sparks. The hot work permit should indicate the location, time, person, on-site supervisor, approver, and fire prevention measures of the hot work.

6.3 库房消防设施要求Requirements for fire protection facilities in warehouses

6.3.1库房应明确消防设施维护管理部门、管理人员及工作职责，建立消防设施值班、巡查、检测、维修、保养、建档等制度，确保消防设施正常运行。

The warehouse should clarify the maintenance and management department, management personnel, and job responsibilities of fire protection facilities, establish a system for fire protection facility duty, inspection, testing, repair, maintenance, and filing, etc., to ensure the normal operation of fire protection facilities.

6.3.2库房禁止擅自关停消防设施。值班、巡查、检测时发现故障，应及时组织修复。

The warehouse is prohibited from shutting down fire-fighting facilities without authorization. When faults are found during duty, inspection, and testing, they should be promptly repaired.

6.3.3库房所设置的消防通道、安全出口、消防车通道，应设置明显标志并保持畅通，不应堆放物品或设置障碍物。

The fire exits, safety exits, and fire truck passages set up in the warehouse should be clearly marked and kept unobstructed, and no items or obstacles should be piled up.

6.3.4库房应设置明显标志划定各类消防设施所在区域，禁止圈占、埋压、挪用和关闭，并应保证该类设施有正常的操作和检修空间。

The warehouse should set up clear signs to delineate the area where various fire-fighting facilities are located, prohibit enclosure, burial, misappropriation, and closure, and ensure that such facilities have normal operating and maintenance space.

6.3.5库房消火栓应有明显标志。消火栓箱不应上锁，箱内设备应该齐全、完好。

The fire hydrant in the warehouse should be clearly marked. The fire hydrant box should not be locked, and the equipment inside the box should be complete and intact.

6.3.6灭火器应设置在位置明显和便于取用的地点，且不得影响安全疏散。

Fire extinguishers should be placed in clearly visible and easily accessible locations,and should not affect safe evacuation.

6.3.7对有视线障碍的灭火器设置点，应设置指示其位置的发光标志。

For fire extinguishers with visual obstacles, illuminated signs indicating their location should be installed.

6.3.8灭火器的摆放应稳固，其铭牌应朝外。手提式灭火器宜设置在灭火器箱内或挂钩、托 架上，其顶部离地面高度不应大于1.50m；底部离地面高度不宜小于0.08m。灭火器箱不得上锁。

The placement of fire extinguishers should be stable, and their nameplates should face outward. Portable fire extinguishers should be installed inside the fire extinguisher box or on hooks or brackets, and their top should not be more than 1.50m above the ground; The height from the bottom to the ground should not be less than 0.08m. The fire extinguisher box shall not be locked.

6.3.9灭火器不宜设置在潮湿或强腐蚀性的地点。当必须设置时，应有相应的保护措施。灭火器设置在室外时，应有相应的保护措施。地处寒区的仓库，寒冷季节要采取防冻措施。

Fire extinguishers should not be placed in damp or highly corrosive locations. When necessary, corresponding protective measures should be taken. When fire extinguishers are installed outdoors, corresponding protective measures should be taken. Warehouses located in cold regions should take anti freezing measures during the cold season.

6.4 应急与演练Emergency and drill

6.4.1 EHS管理负责人可根据数据实际情况定期开展库房相关应急演练，并列入到年度应急救援演练计划中，对应急救援演练情况进行记录。

The EHS management manager may conduct regular emergency drills related to the warehouse based on the actual data situation,and include them in the annual emergency rescue drill plan to record the emergency rescue drill situation.

6.4.2库房发生安全事故后，现场各部门/岗位/业务线有关人员应当立即报告上海浦江数据中心EHS管理负责人。各部门/岗位/业务线任何人发生火灾时应立即报警并立即组织力量扑救。

After a safety accident occurs in the warehouse, relevant personnel from various departments/positions/business lines on site should immediately report to the EHS management person in charge of SHANGHAI PUJIANG Data Center. Any department/position/business line should immediately report a fire to the police and organize forces to extinguish it.

7 附件attachment

无None