危险废物管理制度

Hazardous Waste Management System

内部使用

在使用前，请和文档编写人员确认本文档为最新版本。

未经上海暻熠数据科技有限公司、上海暻耀网络技术有限公司书面许可，其它任何机构或个人不可传阅、引用或复制本文件的内容。

Internal use

Before using, check with the documentation writer to make sure this document is the latest version.

Without the written permission of Shanghai Jingyi Data Technology Co., Ltd., Shanghai Jingyao internet Technology Co., Ltd., no other organization or individual is allowed to circulate, quote or copy the content of this document.

| 文件编号Documents | IP-PJ-EHS-30 |
| --- | --- |
| 版 本Edition | V1.0 |
| 文件维护部门File maintenance department | 上海浦江数据中心 |
| 会签部门Document Signing department | 上海浦江数据中心 |
| 执行范围Scope of execution | 上海浦江数据中心 |

文件修订记录

Document revision record

| 版本号 Edition | 版本日期 Date | 修订内容 Revised Content | 修订人员 Revision personnel | 核准员 Approver | 备注 Remarks |
| --- | --- | --- | --- | --- | --- |
| V1.0 | 2025-06-25 | 初版发布 | 楚靖岳 | 陆磊 |  |

目录 Content

1 目的 Purpose	4

2 适用范围 Scope of application	4

3 规范性引用文件 Refence documents	4

4 术语和定义 Terms and definitions	5

4.1危险废物Hazardous waste	5

4.2贮存Storage	5

4.3贮存设施Storage facilities	5

4.4贮存库Storehouses	5

4.5贮存分区Storage subdivision	5

4.6处置Disposal	5

5 职责Responsibilities	6

5.1 EHS管理负责人EHS Officer	6

5.2各部门/岗位/业务线 All departments/positions/business lines	6

5.3采购部门 Procurement Department	7

6程序 Procedures	7

6.1 危险废物的识别Identification of hazardous waste	7

6.2处置流程 Disposal process	8

6.3危险废物产生、收集、分类与内部转运管理Generation, collection, classification and internal transfer management of hazardous waste	8

6.4危险废物储存管理 Storage and management of hazardous waste	12

6.5危险废物处置管理 Hazardous waste disposal management	14

6.6相关申报Relevant Declarations	14

6.7应急响应 Emergency Response	15

6.8相关标识Relevant signage	16

7 附件 Attachments	20

1 目的 Purpose

本制度旨在建立上海暻熠数据科技有限公司、上海暻耀网络技术有限公司（以下简称“上海浦江数据中心”）危险废物管理流程。目标包括确保遵守国家和地方法律法规、国外相关法规、客户要求以及上海浦江数据中心内部要求，确保危险废物管理合规，降低运营EHS风险，并促进持续改进。

This system aims to establish hazardous waste management process of Shanghai Jingyi Data Technology Co., Ltd., Shanghai Jingyao internet Technology Co., Ltd.. (hereinafter referred to as "SHANGHAI Pujiang Data Center"). Objectives include ensuring compliance with national and local laws and regulations, relevant foreign regulations, customer requirements, and internal requirements of SHANGHAI PUJIANG Data Center, ensuring compliance with hazardous waste management, reducing operational EHS risks, and promoting continuous improvement.

2 适用范围 Scope of application

适用于上海浦江数据中心，以及所有与EHS有关事项的管理职能部门。

Applicable to SHANGHAI PUJIANG Data Center and all management functions related to EHS.

3 规范性引用文件 Refence documents

《中华人民共和国环境保护法》（2014年修订）

Environmental Protection Law of the People's Republic of China (revised in 2014)

《建设项目环境保护管理条例》（2017年10月1日起施行）

Regulations on the Administration of Environmental Protection for Construction Projects (effective October 1, 2017)

《河北省固体废物污染环境防治条例》（2022年12月1日起施行）

Regulations of Hebei Province on the Prevention and Control of Environmental Pollution by Solid Waste (Effective on December 1, 2022)

GB 15562.2   环境保护图形标志 固体废物贮存（处置）场

GB 15562.2   Environmental Protection Graphic symbol of solid waste storage (disposal) site

GB 18597    危险废物贮存污染控制标准

GB 18597 	  Hazardous waste storage pollution control standard

HJ 1259     危险废物管理计划和管理台账制定技术导则

HJ 1259     Technical Guidelines for the development of hazardous waste management plans and management ledgers

HJ 1276     危险废物识别标志设置技术规范

HJ 1276     Technical specification for the setting of hazardous waste identification marks

《国家危险废物名录》（生态环境部、国家发展和改革委员会、公安部、交通运输部、国家卫生健康委员会令第36号公布）

The National Hazardous Waste List (Announced by the Ministry of Ecology and Environment, the National Development and Reform Commission, the Ministry of Public Security, the Ministry of Transport, and the National Health Commission under Order No. 36)

4 术语和定义 Terms and definitions

4.1危险废物Hazardous waste

是指列入国家危险废物名录或者根据国家规定的危险废物鉴别标准和鉴别方法认定的具有危险特性的固体废物。

The term "hazardous waste" refers to the solid waste with hazardous characteristics that is included in the national list of hazardous waste or determined according to the identification standards and methods for hazardous waste prescribed by the State.

4.2贮存Storage

是指将危险废物临时置于特定设施或者场所中的活动。

The activity of temporarily storing hazardous waste in a specific facility or place.

4.3贮存设施Storage facilities

是指专门用于贮存危险废物的设施，具体类型包括贮存库、贮存场、贮存池和贮存罐区等。其中，集中贮存设施是用于集中收集、利用、处置危险废物所附设的贮存危险废物的设施。

It means facilities specially used for the storage of hazardous waste, including storage depots, storage fields, storage pools and storage tank areas. Among them, the centralized storage facility is a storage facility attached to the centralized collection, utilization and disposal of hazardous waste.

4.4贮存库Warehouse

用于贮存一种或多种类别、形态危险废物的仓库式贮存设施。

A warehouse-type storage facility for storing one or more types and forms of hazardous waste.

4.5贮存分区Storage subdivision

一个贮存设施内划分的分类存放危险废物的区域。

A compartmentalized area within a storage facility for the separate storage of hazardous waste.

4.6处置Disposal

是指将固体废物焚烧和用其他改变固体废物的物理、化学、生物特性的方法，达到减少已产生的固体废物数量、缩小固体废物体积、减少或者消除其危险成分的活动，或者将固体废物最终置于符合环境保护规定要求的填埋场的活动。

It means the activities of burning solid waste or altering its physical, chemical and biological characteristics by other means, so as to reduce the amount of solid waste produced, shrink its volume, reduce or eliminate its dangerous components, or the activities of finally placing solid waste in a landfill that meets the requirements of the regulations on environmental protection.

5 职责Responsibilities

5.1 EHS管理负责人EHS Officer

5.1.1负责危险废物分类、贮存和处置的全过程监督管理。

Supervise and manage the whole process of classification, storage and disposal of hazardous waste.

5.1.2负责制定和更新年度危险废物管理计划，并申请官方批准，以满足法规和政府要求。

Be responsible for developing and updating the annual hazardous waste management plan, and apply for official approval to meet regulations and government requirements.

5.1.3建立并更新危险废物储存信息，并联系处置公司进行收集和处置。

Establish and update hazardous waste storage information and contact disposal companies for collection and disposal.

5.1.4参与和实施对危险废物承包商和设施的定期和系统审计，以确保妥善收集和储存以及合法处理、储存和处置。

Participate in and implement regular and systematic audits of hazardous waste contractors and facilities to ensure proper collection and storage and lawful handling, storage and disposal.

5.1.5 组织并实施对不同部门危险废物相关人员的培训，以满足法规和政府要求。

Organize and implement training for hazardous waste related personnel in different departments to meet regulatory and governmental requirements.

5.1.6 负责制定应急响应计划，包括危险废物应急响应行动，并做好应急事件记录。

Responsible for developing emergency response plan, including hazardous waste emergency response actions, and keeping records of emergency events.

5.2各部门/岗位/业务线 All departments/positions/business lines

5.2.1负责上海浦江数据中心运营期间的危险废物产生管理。

Responsible for hazardous waste generation management during the operation of SHANGHAI PUJIANG Data Center.

5.2.2负责做好危险废物收集分类、贴标识、转运至危险废物贮存设施。

Be responsible for the collection, classification, labeling and transfer of hazardous waste to hazardous waste storage facilities.

5.2.3负责向EHS管理负责人提交危险废物处理申请。

Be responsible for submitting hazardous waste disposal application to EHS manager.

5.2.4负责向EHS管理责任人提交新增加的危险废物申请。

Responsible for submitting new hazardous waste application to EHS management person.

5.2.5组织危险废物产生过程应急演练。

Organize emergency drills for hazardous waste generation process.

5.3采购部门 Procurement Department

5.3.1负责与有资质的承包商签订合同，确保危险废物的处理符合相关要求。

Responsible for signing contracts with qualified contractors to ensure that the disposal of hazardous waste meets relevant requirements.

6程序 Procedures

6.1 危险废物的识别Identification of hazardous waste

6.1.1EHS管理负责人对上海浦江数据中心运营期间的产生的危险废物进行识别。

The head of EHS management shall identify the hazardous waste generated during the operation of the SHANGHAI PUJIANG Data Center.

表1数据中心常见的危险废物清单

Table 1 List of common hazardous wastes in data center

| 序号 Number | 废物类别 Category | 危险废物 Waste | 废物代码 Code | 来源 Source | 危险特性 Danger | 有害成分 Ingredient |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | HW31 | 废铅蓄电池 Battery | 900-052-31 | 电池室，柴油发电机等 Battery room, diesel generator, etc | T，C | 铅、硫酸等 Lead, sulfuric acid, etc |
| 2 | HW08 | 废机油 Oil | 900-249-08 | 柴油发电机、冷机等 Diesel generators, chillers, etc | T，I | 石油烃、多环芳烃等 Petroleum hydrocarbon, polycyclic aromatic hydrocarbons, etc |
| 3 |  | 废矿物油 Mineral oil | 900-249-08 | 柴油储罐等 Oil tank | T，I |  |
| 4 | HW06 | 废防冻液 Antifreeze | 900-404-06 | 柴油发电机 Diesel generator | T，I，R | 乙二醇 Glycol |
| 5 | HW49 | 废机油滤芯 Motor oil filter | 900-041-49 | 柴油发电机 Diesel generator | T/In | 石油烃、多环芳烃等 Petroleum hydrocarbons, polycyclic aromatic hydrocarbons, etc |
| 6 |  | 废油滤芯 Oil filter | 900-041-49 | 柴油发电机 Diesel generator | T/In |  |
| 7 | HW49 | 废油桶 Oil bucket | 900-041-49 | 柴油发电机 Diesel generator | T/In | 石油烃、多环芳烃等 Petroleum hydrocarbons, polycyclic aromatic hydrocarbons, etc |
| 8 |  | 废药剂桶 Chemical bucket | 900-047-49 | 加药系统 Dosing system | T/C/I/R | 次氯酸钠等 Sodium hypochlorite, etc |
| 9 |  | 废墨盒 Ink cartridge | 900-041-49 | 办公室 Office | T/In | 铅、水银、镉、铬等 Lead, mercury, cadmium, chromium, etc |
| 10 | HW29 | 废含汞荧光灯管 Light tube | 900-023-29 | 办公室、设备间等 Office | T | 汞 Mercury |

6.2处置流程 Disposal process

6.2.1危险废物整体处置流程包括危险废物产生，收集，分类，内部转运，转运至危废库，以及委托有资质的第三方公司进行处置，所有的环节均需要现场及文字管理记录。

The overall disposal process of hazardous waste includes hazardous waste generation, collection, classification, internal transfer, transfer to hazardous waste storage, and entrusted to a qualified third-party company for disposal. All links require on-site and written management records.

6.2.2危险废物整体处置流程：首先对危险废物的产生各产废部门分类收集后，向EHS管理负责人申请清运及处置。EHS管理负责人指定专人负责转运至危废库，并联络危废处理公司清运。

Overall disposal process of hazardous waste: First of all, after classification and collection of hazardous waste generation departments, apply to the EHS management person for removal and disposal. The person in charge of EHS management shall designate a special person to transport hazardous waste to hazardous waste storage, and contact hazardous waste treatment company for removal and disposal.

6.3危险废物产生、收集、分类与内部转运管理Generation, collection, classification and internal transfer management of hazardous waste

6.3.1上海浦江数据中心需要在完成分类后将危险废物放入单独的容器或包装良好的危险废物中，并在容器上贴上标明危险废物标签。

SHANGHAI PUJIANG Data Center shall put the hazardous waste into separate containers or well-packaged hazardous waste after the completion of classification and label the container with hazardous waste.

6.3.2 危险废物产生情况需要在“固体废物综合管理系统”中进行记录。

The production of hazardous waste needs to be recorded in the "Integrated Solid Waste Management System".

6.3.3危险废物的收集应根据危险废物产生的工艺特征、排放周期、危险废物特性、废物管理计划等因素制定收集计划。收集计划应包括收集任务概述、收集目标及原则、危险废物特性评估、危险废物收集量估算、收集作业范围和方法、收集设备与包装容器、安全生产与个人防护、工程防护与事故应急、进度安排与组织管理等。

Collection of hazardous waste The collection plan shall be formulated according to factors such as the process characteristics of hazardous waste generation, discharge cycle, characteristics of hazardous waste, and waste management plan. The collection plan shall include overview of collection tasks, collection objectives and principles, assessment of hazardous waste characteristics, estimation of hazardous waste collection amount, scope and method of collection operation, collection equipment and packaging containers, production safety and personal protection, engineering protection and emergency response, schedule and organizational management, etc.

6.3.4危险废物的收集应制定详细的操作规程，内容至少应包括适用范围、操作程序和方法、专用设备和工具、转移和交接、安全保障和应急防护等。

Detailed operating procedures shall be formulated for the collection of hazardous waste, including at least the scope of application, operating procedures and methods, special equipment and tools, transfer and handover, safety guarantee and emergency protection.

6.3.5在危险废物的收集和转运过程中，应采取相应的安全防护和污染防治措施，包括防爆、防火、防中毒、防感染、防泄漏、防飞扬、防雨或其它防止污染环境的措施。

In the process of collection and transfer of hazardous waste, appropriate safety protection and pollution prevention measures shall be taken, including explosion-proof, fire-proof, anti-poisoning, anti-infection, anti-leakage, anti-flying, anti-rain or other measures to prevent environmental pollution.

6.3.6 危险废物收集时应根据危险废物的种类、数量、危险特性、物理形态、运输要求等因素确定包装形式，具体包装应符合如下要求：

When collecting hazardous waste, the packaging form shall be determined according to the type, quantity, dangerous characteristics, physical form, transportation requirements and other factors of hazardous waste, and the specific packaging shall meet the following requirements:

- 包装材质要与危险废物相容，可根据废物特性选择钢、铝、塑料等材质。

The packaging material should be compatible with hazardous waste, and steel, aluminum, plastic and other materials can be selected according to the characteristics of waste.

- 性质类似的废物可收集到同一容器中，性质不相容的危险废物不应混合包装。

Wastes with similar properties can be collected in the same container, and hazardous wastes with incompatible properties should not be mixed in packaging.

- 危险废物包装应能有效隔断危险废物迁移扩散途径，并达到防渗、防漏要求。

The packaging of hazardous waste should be able to effectively block the migration and diffusion of hazardous waste, and meet the requirements of anti-seepage and anti-leakage.

- 包装好的危险废物应设置相应的标签，标签信息应填写完整翔实。

The packaged hazardous waste should be equipped with corresponding labels, and the label information should be complete and accurate.

- 盛装过危险废物的包装袋或包装容器破损后应按危险废物进行管理和处置。

After the packaging bags or packaging containers containing hazardous wastes are damaged, they should be managed and disposed of as hazardous wastes.

- 危险废物还应根据GB12463的有关要求进行运输包装。

Hazardous waste should also be transported and packaged in accordance with the relevant requirements of GB12463.

6.3.7危险废物的收集作业应满足如下要求：

The collection operation of hazardous waste shall meet the following requirements:

- 应根据收集设备、转运车辆以及现场人员等实际情况确定相应作业区域，同时要设置作业界限标志和警示牌。

The corresponding operation area should be determined according to the actual situation of collection equipment, transfer vehicles and on-site personnel, and at the same time, operation boundary signs and warning signs should be set.

- 作业区域内应设置危险废物收集专用通道和人员避险通道。

Special channels for hazardous waste collection and personnel escape channels should be set up in the operation area.

- 收集时应配备必要的收集工具和包装物，以及必要的应急监测设备及应急装备。

Collection should be equipped with necessary collection tools and packaging, as well as necessary emergency monitoring equipment and emergency equipment.

- 危险废物收集应做好相关记录，并将记录表作为危险废物管理的重要档案妥善保存。

Relevant records should be made for the collection of hazardous waste, and the record form should be properly kept as an important file for hazardous waste management.

- 收集结束后应清理和恢复收集作业区域，确保作业区域环境整洁安全。

After the collection, the collection operation area should be cleaned up and restored to ensure a clean and safe environment in the operation area.

- 收集过危险废物的容器、设备、设施、场所及其它物品转作它用时，应消除污染，确保其使用安全。

When containers, equipment, facilities, places and other articles that have collected hazardous waste are used for other purposes, they shall be decontaminated to ensure their safe use.

6.3.8危险废物内部转运作业应满足如下要求：

The internal transfer operation of hazardous waste shall meet the following requirements:

- 危险废物内部转运应综合考虑厂区的实际情况确定转运路线，尽量避开办公区和生活区。

The internal transfer of hazardous waste shall comprehensively consider the actual situation of the plant to determine the transfer route, and avoid the office area and living area as far as possible.

- 危险废物内部转运作业应采用专用的工具。

Special tools should be used in the internal transfer operation of hazardous waste.

- 危险废物内部转运结束后，应对转运路线进行检查和清理，确保无危险废物遗失在转运路线上，并对转运工具进行清洗。

After the internal transport of hazardous waste is completed, the transport route should be inspected and cleaned to ensure that no hazardous waste is lost on the transport route, and the transport tools should be cleaned.

6.3.9如果产生新的危废（未列入当前内部危废目录），危废产生部门应通知并向EHS管理负责人提交新增加的危险废物申请表（附件6）。

If a new hazardous waste is generated (not included in the current internal hazardous waste inventory), the hazardous waste generating department shall notify and submit the newly added hazardous waste application form (Attachment 6) to the EHS management manager.

6.3.10 危险废物内部转运前需确保盛装液体或者半固体危险废物的容器保持密封，保持二次盛漏容器盛漏功能完好，以防止废液泄漏，发生废液泄漏或洒落情况，立即通知EHS管理负责人以采取相关应急处置措施。

Before the internal transfer of hazardous waste, it is necessary to ensure that the container containing liquid or semi-solid hazardous waste is kept sealed, and the leakage function of the secondary leakage container is kept intact to prevent the leakage of waste liquid. In case of leakage or spilling of waste liquid, the EHS management person shall be notified immediately to take relevant emergency disposal measures.

6.3.11危险废物收集和转运作业人员应根据工作需要配备必要的个人防护装备，如手套、防护镜、防护服、防毒面具或口罩等。

Hazardous waste collection and transfer operators should be equipped with necessary personal protective equipment according to work needs, such as gloves, protective goggles, protective clothing, gas masks or masks.

6.4危险废物储存管理 Storage and management of hazardous waste

6.4.1所有危险废物产生者应建造专用的危险废物贮存设施，也可利用原有构筑物改建成危险废物贮存设施。

All hazardous waste producers shall build special hazardous waste storage facilities, and can also use the original structures to transform into hazardous waste storage facilities.

6.4.2危废库（含临时存放区）需按规定设置危险废物识别标志。需要对危险废物任何储存容器贴上标签。标签为工作场所、运输或储存场所的员工提供了正确处理各自材料的关键信息，以保护他们自己和环境；并确保在发生紧急情况或事故时做出适当反应。EHS管理负责人每周对危废标识及标签进行检查。

Hazardous waste warehouses (including temporary storage areas) shall be equipped with hazardous waste identification marks in accordance with regulations. Labels shall be affixed to any storage container of hazardous waste. Labels provide employees at the workplace, transport or storage site with critical information to properly handle their respective materials to protect themselves and the environment; And to ensure an appropriate response in the event of an emergency or accident. The EHS management officer inspects hazardous waste signs and labels weekly.

6.4.3在危废库内联合储存某些废物可能不具有化学相容性，在设施规划中必须遵守并考虑足够的储存容量和累积量限制。不相容的危险废物需分开存放，并设有隔离间隔断。EHS管理负责人每周对不相容的危废储存进行检查，检查表详见附件5。

Joint storage of certain wastes in hazardous waste repositories may not be chemically compatible and sufficient storage capacity and accumulation limits must be observed and considered in facility planning. Incompatible hazardous wastes should be stored separately and separated by isolation rooms. The EHS manager shall check the incompatible hazardous waste storage weekly. See Attachment 5 for the check list.

6.4.4 液体危险废物贮存特别通知，容器不得装满。容器顶部与液体表面之间应留有至少100mm高的空间。EHS管理负责人每周对容器内危废储存量进行检查，检查表详见附件5。

Special Notice for Liquid Hazardous Waste storage, containers shall not be filled. There should be a space of at least 100mm high between the top of the container and the liquid surface. The EHS officer shall check the hazardous waste storage capacity in the container every week. See Attachment 5 for the check list.

6.4.5即使液体危险废物储存在室内，也应始终为其提供二次容器以防泄漏，并且确保二次容器处于完好状态。EHS管理负责人每周对盛装危废的二次容器进行检查，检查表见附件5。

Even if liquid hazardous waste is stored indoors, a secondary container should always be provided to prevent leakage and ensure that the secondary container is in good condition. The EHS officer shall inspect the hazardous waste secondary container on a weekly basis. See Attachment 5 for the checklist.

6.4.6盛装危险废物的容器材质和衬里要与危险废物相容（不相互反应）。EHS管理负责人每周对盛装危废的二次容器进行检查，检查表详见附件5。

The material and lining of the container containing hazardous waste shall be compatible with hazardous waste (do not react with each other). The EHS management person shall inspect the secondary containers containing hazardous waste every week. See Attachment 5 for the check list.

6.4.7危废库存放的危废需按危险性质分类存放，并确保和普通废物无混放现象。EHS管理负责人每周对危废分类存放进行检查，检查表详见附件5。

The hazardous waste stored in the hazardous waste inventory shall be classified according to the hazardous nature, and ensure that there is no mixing with ordinary waste. The person in charge of EHS management shall check the classified storage of hazardous waste on a weekly basis. See Attachment 5 for the check list.

6.4.8危废仓库地面与裙需用坚固、防渗的材料建造，且表面无裂隙。

The floor and skirt of the hazardous waste warehouse shall be made of solid and impermeable materials with no cracks on the surface.

6.4.9危废仓库是否设置了堵截泄漏的裙脚，并且地面与裙脚所围建的容积不低于堵截最大容器的最大储量或总储量的五分之一。EHS管理负责人每月对危废库地面及裙脚进行检查，检查表详见附件5。

Whether the hazardous waste warehouse is equipped with skirt feet to block leakage, and the volume built by the ground and skirt feet is not less than the maximum reserve or one-fifth of the total reserve of the largest blocked container. The person in charge of EHS management shall inspect the ground and skirt feet of the hazardous waste warehouse every month. See Attachment 5 for the inspection list.

6.4.10在常温常压下易爆、易燃及排出有毒气体的危险废物必须进行预处理（比如进行冷却降温），使之稳定后贮存，否则，按易爆、易燃危险品贮存。

Hazardous wastes that are explosive, flammable and discharge toxic gases at normal temperature and pressure must be pre-treated (such as cooling and cooling) to make them stable and stored; otherwise, they should be stored as explosive, flammable and dangerous goods.

6.4.11危废库要有安全照明设施和观察窗口。

Hazardous waste storage shall be provided with safe lighting facilities and observation Windows.

6.4.12现场应急计划还应包括如何处理紧急情况或异常情况产生的危险废物的说明。将应急响应现场处置方案放在现场。

The on-site emergency plan should also include instructions on how to deal with hazardous waste generated in an emergency or abnormal situation. Place the emergency response site disposal plan on site.

6.4.13确保通风系统能够正常运行，并定期对系统进行检查和维护。EHS每月对危废库通风及静电系统进行检查，检查表详见附件5。

Ensure that the ventilation system is able to operate properly, and regularly inspect and maintain the system. EHS shall check the ventilation and electrostatic system of hazardous waste warehouse every month. See Attachment 5 for the check list.

6.5危险废物处置管理 Hazardous waste disposal management

6.5.1验证有效执照/许可证/批准或政府要求的其他必要证书，以处理相关危险废物。

Verify valid licenses/permits/approvals or other necessary certificates required by the government to handle relevant hazardous waste.

6.5.2现场确认承包商设施运行的环境性能及其符合如下最低要求：

Verify on site the environmental performance of the operation of the Contractor's facilities and their compliance with the following minimum requirements:

- 目视检查运输车辆、装卸区、储存设施、加工技术和设备。

Visual inspection of transport vehicles, loading areas, storage facilities, processing technology and equipment.

- 确认员工和经理的能力、资格计划和记录。

Verify employee and manager competencies, qualification plans and records.

- 研究公司关键操作的组织，例如过程描述和工作说明中描述的任务和职责的内部监督程度检查以及材料测试或维护程序（测试和检查间隔、已知危险的防护和消除、应急准备等）。

The organization that studies the critical operations of the company, such as the degree of internal oversight checks of the tasks and responsibilities described in the process description and job description, and the material testing or maintenance procedures (test and inspection intervals, protection and elimination of known hazards, emergency preparedness, etc.).

6.5.3检查与危险废物相关的装载、运输和卸载活动，尤其是危险废物包装和适当的标签。

Check the loading, transportation and unloading activities related to hazardous waste, especially hazardous waste packaging and proper labeling.

6.5.4确保与合格公司签订的合同载明了详细的权利和义务，如有任何变更，应及时更新合同。

Ensure that contracts with qualified companies set out detailed rights and obligations and should be updated in a timely manner if there are any changes.

6.6相关申报Relevant Declarations

6.6.1数据中心应当通过当地固体废物动态信息平台系统在线填写并提交当年度的危险废物管理计划及危险废物转移情况信息。由在系统中自动生成备案编号和回执，完成备案。系统操作界面见附件1。

Data Center shall fill in and submit the annual hazardous waste management plan and hazardous waste transfer information online through the local Solid Waste Dynamic Information Platform system. The filing number and receipt will be automatically generated in the system to complete the filing. See Attachment 1 for the system operation interface.

6.6.2若上海浦江数据中心存在新改扩建项目，且项目造成固废（危废）种类发生变化，或者固废（危废）处置方式发生变化，固废（危废）产生量发生重大变化，企业需要根据相关法规要求重新报批环境影响评价报告，以及更新排污许可证，也需要更新危废管理计划。

If there is a new renovation and expansion project of SHANGHAI PUJIANG Data Center, and the type of solid waste (hazardous waste) changes as a result of the project, or the disposal method of solid waste (hazardous waste) changes, and the production volume of solid waste (hazardous waste) changes significantly, the enterprise needs to submit the environmental impact assessment report for approval again according to relevant laws and regulations, renew the emission permit, and update the hazardous waste management plan.

6.6.3企业需要保持与政府相关部门的沟通，及时掌握政府监管部门的检查趋势及检查计划等信息，为迎接政府检查做好充分准备。

The enterprise needs to maintain communication with relevant government departments, timely grasp the inspection trend and inspection plan information of government regulatory departments, and make full preparations for government inspections.

6.6.4在编制突发环境事件应急预案时，需要有危险废物应急处置相关内容，应急预案根据法规及政府相关要求进行备案，并定期开展危险废物应急演练。

When preparing the emergency plan for environmental emergencies, it is necessary to include the relevant contents of the emergency disposal of hazardous waste. The emergency plan shall be recorded in accordance with laws and regulations and the relevant requirements of the government, and hazardous waste emergency drills shall be carried out regularly.

6.7应急响应 Emergency response

6.7.1数据中心应制定符合本中心要求的危险废物应急预案。

The data center shall formulate an emergency plan for hazardous waste that meets the requirements of the Center.

6.7.2数据中心应每年进行危险废物相关演练。

The data center shall conduct hazardous waste related drills every year.

6.7.3危险废物溢出/泄漏时，应采取以下具体措施：

When hazardous waste spills/spills, the following specific measures should be taken:

- 任何人员发现任何危险废物相关事故时，应立即向EHS管理负责人报告。

Any person who discovers any hazardous waste related incident shall immediately report it to the EHS officer.

b ) 经培训合格的数据中心人员处置危险废物溢出/泄漏前，应根据要求穿戴合适的PPE。

Trained and qualified data center personnel should wear appropriate PPE as required before disposing of hazardous waste spills/spills.

6.8相关标识Relevant signage

6.8.1危险废物标签详见图1，标签要求如下：

The labels of hazardous waste are shown in Figure 1. The label requirements are as follows:

- 危险废物标签 颜色：危险废物标签背景色应采用醒目的橘黄色，RGB 颜色值为（255, 150, 0）。标签边 框和字体颜色为黑色，RGB 颜色值为（0, 0, 0）。

Hazardous waste label color: The background color of the hazardous waste label should be eye-catching orange yellow, and the RGB color value is (255, 150, 0). The label frame and font color should be black and the RGB color value should be (0, 0, 0).

- 字体：危险废物标签字体宜采用黑体字，其中“危险废物”字样应加粗放大。

Font: The font of the hazardous waste label should be in bold, and the word "hazardous waste" should be bolded and enlarged.

- 尺寸：危险废物标签的尺寸宜根据容器或包装物的容积按照表 2 中的要求设置。

Size: The size of the hazardous waste label should be set according to the volume of the container or package according to the requirements in Table 2.

- 材质：危险废物标签所选用的材质宜具有一定的耐用性和防水性。标签可采用不干胶印刷品，或印刷品外加防水塑料袋或塑封等。

Material: The material selected for the hazardous waste label should have certain durability and water resistance. Labels can be self-adhesive printed matter, or printed matter plus waterproof plastic bag or plastic seal.

- 印刷：危险废物标签印刷的油墨应均匀，图案和文字应清晰、完整。危险废物标签的文字边缘宜加黑色边框，边框宽度不小于1 mm，边框外宜留不小于3 mm的空白。

Printing: The ink of hazardous waste label printing should be uniform, and the pattern and text should be clear and complete. The text edge of the hazardous waste label should be added with a black border, the width of the border should be no less than 1 mm, and the space outside the border should be no less than 3 mm.

图1 危险废物标签示例

Figure 1 Example of hazardous waste label

表2危险废物标签的尺寸要求

Table 2 Size requirements for hazardous waste labels

| 序号 No. | 容器或包装容积 Volume (L) | 标签最小尺寸 Size (mm*mm) | 最低文字高度 Height (mm) |
| --- | --- | --- | --- |
| 1 | ≤50 | 100*100 | 3 |
| 2 | ＞50~≤450 | 150*150 | 5 |
| 3 | ＞450 | 200*200 | 6 |

6.8.2上海浦江数据中心的贮存设施应在设施附近或场所的入口处设置相应的危险废物贮存设施标志。危险废物贮存设施标志详见图2，标志具体要求如下：

The storage facility of SHANGHAI PUJIANG Data Center shall set up the corresponding hazardous waste storage facility sign near the facility or at the entrance of the site. See FIG. 2 for details on the marks of hazardous waste storage facilities, and the specific requirements of the marks are as follows:

- 材质：危险废物贮存设施标志宜采用坚固耐用的材料（如 1.5 mm～2 mm 冷轧钢板），并做搪瓷处理或贴膜处理。一般不宜使用遇水变形、变质或易燃的材料。柱式标志牌的立柱可采 用 38×4 无缝钢管或其他坚固耐用的材料，并经过防腐处理。

Material: Hazardous waste storage facility signs should be durable materials (such as 1.5 mm ~ 2 mm cold rolled steel), and do enamel treatment or film treatment. Generally, it is not appropriate to use materials that are deformed, metamorphic or flammable in case of water. The column of the column sign can be made of 38×4 seamless steel pipe or other durable materials, and after anti-corrosion treatment.

- 印刷：危险废物贮存设施标志的图形和文字应清晰、完整，保证在足够的观察距离条件下不影响阅读。三角形警告性图形与其他信息间宜加黑色分界线区分，分界线的宽度宜不小于 3 mm。

Printing: The graphics and text of the hazardous waste storage facility sign should be clear and complete, to ensure that it does not affect the reading under the condition of sufficient observation distance. A black dividing line should be added between the triangular warning graphics and other information, and the width of the dividing line should not be less than 3 mm.

- 外观质量：危险废物贮存设施的标志牌和立柱无明显变形。标志牌表面无气泡，膜或搪瓷无脱落。图案清晰，色泽一致，没有明显缺损。

Appearance quality: signs and columns of hazardous waste storage facilities have no obvious deformation. The surface of the sign has no bubbles, and the film or enamel has not fallen off. The pattern is clear, the color is consistent, and there is no obvious defect.

图2 危险废物贮存设施标志示例

Figure 2 Example of signs for hazardous waste storage facilities

表3 不同观察距离时危险废物贮存设施标志尺寸要求

Table 3 Size requirements of hazardous waste storage facility signs at different viewing distances

| 设置位置 Location | 观察距离 Distance L（m） | 标志牌整体外形最小尺寸 Size（mm） | 三角形警告标志 Triangle warning sign |  |  | 最低文字高度 Text height |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 三角形外表a1(mm) | 三角形内长a2（mm） | 边框外交圆弧半径（mm） | 设施类型名称 Facility name | 其他文字 Other text |
| 露天/室外入口 Entrance | ＞10 | 900*558 | 500 | 375 | 30 | 48 | 24 |
| 室内 Indoor | 4＜L≤10 | 600*372 | 300 | 225 | 18 | 32 | 16 |
| 室内 Indoor | ≤4 | 300*186 | 140 | 105 | 8.4 | 16 | 8 |

6.8.3危险废物贮存分区标志详见图3，具体要求：

See Figure 3 for the hazardous waste storage zoning mark, with specific requirements:

- 颜色：危险废物分区标志背景色应采用黄色，RGB 颜色值为（255, 255, 0）。废物种类信息应采用醒目的橘黄色，RGB 颜色值为（255, 150, 0）。字体颜色为黑色，RGB 颜色值为（0, 0, 0）。

Color: The background color of the hazardous waste zoning sign should be yellow, and the RGB color value is (255, 255, 0). The waste type information should be in striking orange with RGB color values of (255, 150, 0). The font color should be black with an RGB color value of (0, 0, 0).

- 字体：危险废物分区标志的字体宜采用黑体字，其中“危险废物贮存分区标志”字样应加粗放大并居中显示。

Font: The font of the hazardous waste zoning sign should be in boldface, and the words "Hazardous waste storage Zoning sign" should be bolded and enlarged and displayed in the center.

- 尺寸：危险废物贮存分区标志的尺寸宜根据对应的观察距离按照表 4中的要求设置。

Size: The size of the hazardous waste storage zone sign should be set according to the requirements in Table 4 according to the corresponding viewing distance.

- 材质：危险废物贮存分区标志的衬底宜采用坚固耐用的材料，并具有耐用性和防水性。废物贮存种类信息等可采用印刷纸张、不粘胶材质或塑料卡片等，以便固定在衬底上。

Material: The substrate of the hazardous waste storage zone sign should be made of strong and durable material with durability and water resistance. The type of waste storage information can be printed paper, non-stick materials or plastic cards, etc., in order to be fixed on the substrate.

- 印刷：危险废物贮存分区标志的图形和文字应清晰、完整，保证在足够的观察距离条件下不影响阅读。“危险废物贮存分区标志”字样与其他信息宜加黑色分界线区分，分界线的宽度不小于2 mm。

Printing: The graphics and text of the hazardous waste storage zone sign should be clear and complete to ensure that it does not affect reading under the condition of sufficient observation distance. The words "hazardous waste storage zone sign" and other information should be distinguished by a black dividing line, and the width of the dividing line is not less than 2 mm.

图3 危险废物贮存分区标志

Figure 3 Hazardous waste storage zone sign

表4 危险废物贮存分区标志的尺寸要求

Table 4 Size requirements for hazardous waste storage zone signs

| 观察距离 Distance(m) | 标志整体外形最小尺寸Size（mm） | 最低文字高度 Height(mm) |  |
| --- | --- | --- | --- |
|  |  | 贮存分区标志 Storage zone sign | 其他文字 other |
| 0＜L≤2.5 | 300*300 | 20 | 6 |
| 2.5＜L≤4 | 450*450 | 30 | 9 |
| L＞4 | 600*600 | 40 | 12 |

7 附件 Attachments

- IF-PJ-EHS-3001《固体废物综合管理系统操作界面》 Hazardous Waste Management System

- IF-PJ-EHS-3002《危险废物产生环节记录表》Hazardous Waste Generation Record

- IF-PJ-EHS-3003《危险废物入库环节记录表》Hazardous Waste storage Record

- IF-PJ-EHS-3004《危险废物出库环节记录表》Hazardous Waste Delivery Record

- IF-PJ-EHS-3005《危险废物储存检查表》Hazardous Waste Storage Inspection Record

- IF-PJ-EHS-3006《新增危废申请表》New Hazardous Waste Application Record